제조공정도

(임스 리프레쉬 오일)

Mixing

1. Mentha Arvensis(peppermint) Leaf Oil : 66%

2. Rosmarinus Officinalis(Rosemary) Leaf Oil : 4%

3. Juniperus Communis Fruit Oil : 4%

4. Eucalyptus Globulus Leaf Oil : 5%

5. Lavandula Angustifolia(Lavender) Oil : 9.5%

6. Cupressus Sempervirens Leaf Oil : 10%

Microfludizer

Hydrosol distilled from Mentha Arvensis(peppermint) Leaf : 0.5%~5% of base oil

(Yield between 100~105%)

Microfludizer condition :

200~500 bar (197~493 atm), 1 time

Mixing condition :

Room Temp, 100rpm, Total 1min

Mix. Product 1

(Base Oil)

High pressure dispersed 
nano emulsion

고압분산에멀젼 허브복합정유