Manufacturing Process

(Liposome Acetyl Hexapeptide-AJ1)

Water:                    87.95 % (w/w)

Glycerin:                   10.0 % (w/w)

1,2-Hexanediol:             1.0 % (w/w)

Acetyl Hexapeptide-8:        0.05 % (w/w)

Hydrogenated Lecithin:      1.0 % (w/w)

Mixing

Homo Mixing Condition:

80 ℃, 3,000rpm, Total 10 min

Microfludizer

Microfludizer Condition: 1,000 bar

Liposome Acetyl Hexapeptide-AJ1